Error Model Description

The repeatability of each gene’s measured expression change across multiple nominally identical independent experiments is the strongest input to the assignment of confidence value.  However, in order to average repeated experiments with appropriate relative weights, a model for the uncertainties in individual array experiments is required.  Also, there is a problem dealing with log(expression) when background-subtracted intensities corresponding to weakly expressed genes can be negative.

We have adopted a single-array error model form which solves these two problems.  Although inelegant, it fits the errors and their physical origins better than the ratio distribution suggested in http://www.nhgri.nih.gov/DIR/LCG/15K/HTML/img_analysis.html .

It assumes a combination of additive and multiplicative error components in both channels of a two-color experiment.  The statistic used to define significance is

X = (a2 - a1) / [12 + 22 + f 2 (a12 + a22 )]1/2				(1)

where a1,2 are the intensities measured in the two channels for each spot, 1,2 are the uncertainties due to background subtraction, and f is a fractional multiplicative error such as would come from hybridization non-uniformities, fluctuations in the dye incorporation efficiency, scanner gain fluctuations, etc.  We have found that X is approximately normal.  and f are chosen so that X has unit variance and tracks the increase in fractional error toward lower-intensity spots – see Figure 1.  tends to vary from array to array and so is derived for each array based on the background fluctuation level.  f  tends to be fairly constant from array to array and is determined from control experiments where nominally the same sample is hybridized in both channels.

The two-sided probability for an observed regulation of magnitude |X| is then

P-Value = 2(1 - Erf(|X|))					(2)

For display and array averaging purposes it is useful to express measurements and errors in terms of log10(a2/a1).  This requires somewhat arbitrarily repairing negative values to a small positive value equal to a scanner raw count, and capping the resulting ratios at 100.   The uncertainty in the log(Ratio) is then defined as

 log10(a2/a1) = log10(a2/a1) / X					(3)

and this allows weighted combining of multiple observations of log(Ratio).  Measurements which come from faint spots near the background will have small values of X, large errors, and will be given low weight when combining repeated measurements.

Linear averaging of intensities, rather than averaging of the log(Ratio), is possible, but leads to poorer accuracy in technologies where the absolute intensity calibration is uncertain from array to array.

We use the minimum-variance weighted average to compute the mean log10(a2/a1) of each reported gene:

wi=1σi2                        (4)

x=i=1,nwixiwi                     (5)

Here i is the error of log10(a2/a1), xi stands for i-th measurement of log10(a2/a1), n is the number of repeats.

The error of xcan be computed in two ways.  One is to propagate the errors i , another is from the scatter of xi:

σp2=1wi                     (6)

σs2=1(n−1)wiwi(xi−x)2              (7)

The propagation error p relies totally on the error estimation of each individual slide, and therefore is subject to bias or systematic uncertainties. Whereas the error from the scatter of the data, s is an unbiased measure, but has large fluctuations when the number of repeats is small. Ideally, we would like to use p when there is only one measurement, and gradually shift to s when the number of repeats is large. We can accomplish that by doing a weighted mean of error. Statistically, the error of s is expressed as:

σσs=σs2(n−1)                         (8)

From Eq.8 we can conclude that the weight for combining is proportional to n. We therefore combine two errors as:

σx=σp+(n−1)σsn              (9)

For the average of multiple slides, the significance statistic X  is computed as:

X=xσx                                (10)

and the confidence is computed using Equation (2) above.

X = +2

X = -2

Figure 1.   Single-array error model contour fits control (same vs. same) two-color hybridization data.  Yeast full-genome array hybridized with wild-type strain samples labeled independently in two different fluors.  Horizontal axis is mean of log(Intensity) of the two channels.